Pukepuke Lagoon concession WA219

Management Plan

Table of Contents

1.	Pukepuke Lagoon	3

1.1 Background	3

1.2 Potential environmental/ecological impacts of the weir and spillway	4

Ecological assessment of drainage channels – April 2025	5

1.3 Concession activity	6

2.	Spray Management	6

2.1 Purpose	6

2.2 Approved Chemicals	6

2.3 Spray area	6

2.4 Notification	6

3.	Vegetation Clearance	7

3.1 Mechanical clearance	7

3.2 Controls	7

3.3 Timeframes	7

3.4 Notification	8

4.	Existing Weir	8

4.1 Purpose	8

4.2 Weir condition	8

4.3 Fish passage	8

5.	Landcorp	8

6.	Monitoring	9

6.1 Weed documentation	9

6.2 Eel numbers	9

6.3 Swan numbers	9

6.4 Water level monitoring	9

6.5 Inspections	10

6.6 Annual reporting	11

Appendix A – Easement Area Plan

# Pukepuke Lagoon

## 1.1 Background

Pukepuke Lagoon is a coastal dune lake situated between Himatangi and Tangimoana west of Puke Puke Road. A straightened channel that drains surrounding farmland enters the site on the eastern boundary and continues to the lake body. Other farm drains to the south flow into the lake from outside the site boundary. The outlet of the lake is channelised and flows northward from the northern shore of the lake before a 90° bend westward directs the channel along the northern edge of the wetland vegetation. Another elbow bend redirects the channel through wetland vegetation before leaving the site. The channel then continues for another 2.6km through radiata pine forest to the coast. The lake is connected to the coast by less than 4km of straightened stream channel and provides an important pathway for indigenous fish to the ocean.

The lake is shallow, reaching a maximum depth of approximately 2m and occasionally drying out during summer droughts. Over the last century the lake has reduced in size from 160ha to approximately 15ha due to the effects of sand drift, changes to local drainage and the spread of raupō.

Pukepuke is a stewardship area under the Conservation Act with a desired outcome that the conservation area is conserved to a healthy functioning state by 2030. Therefore highlighting the need for an environmental management plan from those using the site.

(Figure 1: Drainage channels within the Pukepuke Lagoon Conservation area)

A series of farm drains from the Oroua Downs Drainage Scheme drain into the lake and then out towards the coast as seen above in Figure 1. However, historical issues with inundated farmland prior to the installation of the weir continue today, this flooding occurs due to the overgrowth of vegetation specifically raupō within the inlet and outlet channels of the lagoon which restricts the flow of water out from the lake during periods of high rainfall and causes it to back up and overflow into surrounding farmland, particularly to the south of the lake. This flooding can have a detrimental impact on the landowner as the productive land becomes unusable, with significant die-back of pasture, causing a loss of productivity. Those who own or occupy the flood prone land have concerns around the concession document only permitting the mechanical clearance of vegetation to occur every 5 years as it is believed that this allows for the overgrowth of vegetation within the drains to the extent where it cannot be controlled adequately through annual spraying. Along with this, the concession document only requires the Council to record when flooding or ponding occurs on the adjacent Landcorp property, with arrangements to be in place for Landcorp to record this by taking photos at set locations and reading the gauges.

The proposal to construct the additional fixed height weir on the outlet drain arose to facilitate the drainage of stormwater from agricultural land surrounding the lagoon by bypassing a constricting reach of the outlet drain channel. A “historically acceptable” lake level height of 6.33m.a.s.l was sought as this was believed to stop water from ponding on the farmland long enough to contribute to these issues.

The desired purpose of the fixed weir includes:

- Lowering the lake level more rapidly during flood events or periods. The effectiveness of this is dependent on the unimpeded flow of water from the main lagoon to the weir along the existing drains.

- Contributing to more rapid draining of surface water from farmland immediately adjacent to the Pukepuke inlet drain. The effectiveness of this is dependent on the unimpeded flow of water along that drain and into the main lagoon.

Therefore, the weirs performance is likely being constrained by raupō overgrowth in the drains to the lagoon outlet and consequently adjacent farmland continues to experience flooding during periods of sustained or intense rainfall.

This Management Plan is to be read in conjunction with the Department of Conservation (DOC) Natural Heritage Plan for Pukepuke Lagoon, Manawatū. The management goals outlined in the Natural Heritage Plan include the following:

- Co-management where key stakeholders work in a partnerships to achieve agreed outcomes

- Pest control including the control and prevention of pest plant and animal species

- Habitat connectivity including enhancing the ecological function of existing habitats

- Quality of terrestrial and aquatic habitats.

## 1.2 Potential environmental/ecological impacts of the weir and spillway

A statement of likely ecological environmental impacts of the additional fixed weir on the lagoon produced by Murray Williams of DOC for MDC prior to the installation of the weir stated the following:

- The reduction of the duration of high water levels is not likely to impact upon the lagoon as habitat for bittern and dabchick. Winter feeding opportunities for paradise shelduck, swan and ducks on areas of flooded pasture immediately adjacent to the reserve could be limited but this would be inconsequential given that other areas of flooded land will persist nearby. However, there could be some impact on the provision of nest sites for dabchick and swan if, at the commencement of their breeding time the majority of the raupō stands are not inundated.

- Eels are known to make use of the flooded fields adjacent to the inlet drain and the reserves north eastern boundary for feeding. However this is likely an opportunistic feeding site for eels and the removal of the flooded pasture is unlikely to be a significant factor affecting the abundance of eels in the reserve and the welfare of the eel population.

- Rapidly lowering winter water levels would have the biggest impact on subsequent summer levels.

- Extensive drying of the lake to levels below 5.9m.a.s.l significantly increases duck and swan mortality in late summer through limiting feeding opportunity and making the ingestion of spent lead shot more likely. Very low lake levels will also impact upon the resident eel population.

### Ecological assessment of drainage channels – April 2025

In April 2025 an updated ecological assessment was provided by Aaron Madden, Biodiversity Project Manager of Green by Nature. This ecological report was requested by MDC prior to undertaking any mechanical clearance of overgrown vegetation within the easement area to identify and provide the following:

- Invasive weeds

- Note any pest aquatic fish (carp)

- A general description of the vegetation within the easement area

- Recommendations around the use of spray to control invasive plant species (if applicable).

Recommendations from the ecological assessment included:

- Mechanical clearing would be sufficient to keep the easement drains open until the next round of mechanical clearing, there are no weed species present that demand chemical treatment.

- Chemical control should be considered for the field horsetail at the western end of the inlet drain before it reaches the margin of the lagoon. This would require multiple treatments per year over a couple of years and therefore this factors into the decision on whether this should be controlled in this manner.

- During mechanical clearance, efforts should be made to protect the native species present including:

- Carex geminata (cutty grass) – located on the northern side of the track along the perimeter drain. It is advised to keep any mechanical drain cleanings on the southern side of the track along this section.

- Bolboschoenus fluviatilis (marsh clubrush, kukuraho, pūrua grass) – plants of this species growing within the bed of the drain can be removed but effort should be made to protect the individuals growing on the banks.

- Carex secta (purei)

- Coprosma propinqua (mingimingi) – growing next to come cabbage trees near the weir

- Cyperus ustulatus (giant umbrella sedge)

- During a site visit on 7 April 2025 MDC and Green by Nature discussed planting taller native species along one side of the drains to provide shade. This approach would help suppress weed growth and allow for longer periods between mechanical clearance.

- Planting of the northern side of the inlet drain and the western side of the outlet drain is likely to have a significant effect on regrowth of water celery. However, there is limited scope for planting the perimeter drain given the access for mechanical clearing operations is from the northern side, which is the side required for shade planting.

- Suitable native tree species identified for the site include kahikatea, kōhūhū, narrow-leaved lacebark, and mingimingi following suitable eco-sourcing protocols to ensure success.

- The fish passage structure at the weir required some repair work, this work was completed during the mechanical clearance.

- There is the potential need to look at the way the entire site is managed due to it becoming increasingly difficult to control the natural sequence of a coastal lake ecosystem becoming a wet forest ecosystem. Controlling undesirable species while encouraging desirable species would ensure the site is not overrun with weedy exotic species.

## 1.3 Concession activity

MDC hold an easement concession (WA-219) granted by DOC which allows MDC the right to access the Pukepuke Lagoon conservation area to undertake the following:

- Carry, convey, lead, drain or discharge stormwater in a free and unimpeded flow and in any quantity across the easement area;

- Use, maintain and repair the existing weir at a height of 6.32m.a.s.l and;

- Construct, use and maintain a spillway.

And for the above purposes MDC shall have the right:

- To enter and re-enter the Easement Area on foot or by any reasonable mode of transport;

- To carry out on or in the Easement Area such works as are necessary for the exercise of the powers and authorities granted by the Easement, and;

- To remove as necessary natural vegetation affecting MDC’s use of the easement.

# Spray Management

## 2.1 Purpose

To clear raupō or weeds which encroach on the drain.

The concession states that the spraying of raupō can occur annually between 1 January and 31 March.

## 2.2 Approved Chemicals

Spraying will only be performed with chemicals approved by both DOC and Horizons Regional Council. This list has not been provided as DOC has discouraged the use of sprays within the lagoon area as a blanket vegetation control method. DOC and iwi approval will be obtained prior to utilising sprays in the lagoon drains.

## 2.3 Spray area

- The concession states that spraying may only be undertaken within the present inlet and outlet channels of the easement area.

- There will be no spraying in the lagoon or any associated wetlands. The spraying does not extend into the Coastal Marine Area.

- Spraying will only take place downstream of the lagoon, with no direct spraying to the lagoon. Spraying will take place in calm conditions to minimise and spray drift entering the lagoon.

- Factors which will be considered prior to spraying include wind strength and direction and rain. Strong winds can increase the risk of spray drift and rain would decrease the effectiveness of the spraying.

## 2.4 Notification

MDC will notify DOC 28 days prior to any spraying commencing on the site to allow DOC to coordinate the spraying with any other operations being undertaken at the site at the same time.

# Vegetation Clearance

## 3.1 Mechanical clearance

MDC may mechanically clear vegetation from the channels in the easement area once every five years. Mechanical clearing of vegetation from the channel will be undertaken with a weed bucket. MDC will ensure that no material is deposited on any tracks, a joint inspection by MDC and DOC may be undertaken after any mechanical clearance has been completed to ensure compliance if requested by DOC. If material has been deposited on any tracks MDC will, at its own expense, remove it from the track.

During mechanical clearance channels will not be widened past the present width of the weir (6 metres) nor will the channels be deepened.

If at any time DOC considers the clearing of raupō within the Easement Area is having an undue effect on the eel population then changes to MDC’s raupō clearing operation may be requested by DOC. These changes must be implemented as soon as practicable.

## 3.2 Controls

Mechanical vegetation removal will be subject to the following controls based on conditions within the concession and concerns expressed during a combined site visit between MDC, DOC and iwi:

- A weed bucket will be used to remove the vegetation

- A site walk over must be completed prior to the clearance to identify the pest control sites, these should be clearly marked if required to avoid unintentional damage or loss

- Vegetation removal will only occur in the area covered by the concession

- No vegetation will be deposited on the tracks

- Subject to DOC approval, hollows and ruts within the access track can be filled with sediment to improve access

- The width of channel shall not be increased beyond 6 metres

- The channel shall not be deepened by removing the natural bed material. This allows accumulated material to be removed to reinstate the channel capacity

- All machinery and hydraulic equipment will be checked prior to commencing work

- Any refuelling of machinery will occur outside of the DOC conservation land

- Any vegetation removed will be placed so it cannot re-enter the waterway.

## 3.3 Timeframes

The concession specifies that MDC may mechanically clear vegetation only during the period beginning 1 January and ending 31 March, once every 5 years or more frequently with written approval from DOC. Decisions by DOC as to whether to allow more frequent mechanical clearance may involve an assessment of the following factors:

- The rate and extent of raupō growth

- Whether raupō growth has prohibited water movement through the lagoon, to the extent that water flow over the spillway is impeded

- Whether raupō growth is such that annual chemical spraying is unlikely to achieve the desired result

## 3.4 Notification

MDC will notify DOC of any intended mechanical clearing of the channels at least 28 days prior to commencement.

# Existing Weir

The existing weir will be maintained by MDC so that it is not likely to fail nor allow water to seep through it during the months of January, February and March.

## 4.1 Purpose

The purpose of the fixed weir is to control the flow of water from the outlet of the lagoon. This is designed to maintain the water levels within the lagoon at a height of 6.32m.a.s.l specifically following significant flood producing storm events to avoid excessive and attenuated flooding of farmland around the perimeter of the lagoon and contribute to more rapid draining of surface water from farmland immediately adjacent to the inlet drain. The effectiveness of the weir in achieving this purpose is dependent on the unimpeded flow of water from the main lagoon to the weir along existing drains at both the inlet and outlet. Both historic and recent records show that the effectiveness of the weir has been impacted by the overgrowth of vegetation specifically raupō within the outlet drain, allowing and exacerbating the flooding of adjacent farmland.

## 4.2 Weir condition

If the existing weir fails MDC will immediately replace it or dam the channel to ensure that the lagoon does not drain.

## 4.3 Fish passage

The design of the fish passes were assessed by DOC following the commencement of the concession, following on from this MDC will maintain the fish passage and monitor its condition during bi-monthly site visits.

# Landcorp

Landcorp Farming Limited owns the farmland surrounding the northern, eastern and western areas of the lagoon and conservation area. An easement exists on this property to allow the Pukepuke Lagoon area to be accessed.

It is proposed that MDC will arrange for Landcorp Farming Limited to make note when flooding or ponding occurs on the property, this is to be done by the property owners of Landcorp Farming taking photos during periods of inundation at set locations, along with reading of the gauges at this time.

# Monitoring

MDC will, at its own cost, carry out baseline monitoring during the term of the concession at regular intervals, so as to ascertain the following:

- Whether aggressive weed plants and weeds colonise the zone receiving less inundation

- Whether the nesting numbers of dabchick and swan within the reserve are adversely affected by the spillway

- Whether the eel population within the reserve is adversely affected by the clearance of channel vegetation

## 6.1 Weed documentation

MDC will photograph the margin of the lagoon between the two drains and the margins of the small lake annually to monitor weed colonisation.

If at any time DOC considers weed colonisation to be problematic MDC will pay for any new weed control measures which DOC deems necessary for controlling such weeds.

## 6.2 Eel numbers

The concession requires that MDC, on an annual basis identify trends in eel numbers, in particular the quantities caught, the ease by which they are caught and estimations about whether numbers are increasing or decreasing.

As Ngāti Apa and Rangitane o Manawatū are kaitiaki and regularly catch eels from the lagoon MDC have reached out to representatives of each iwi in an attempt to reach a joint agreement on who should undertake the eel counts and how. However, this agreement is yet to be finalised.

## 6.3 Swan numbers

MDC will obtain annual survey information on swan numbers for the lagoon from the relevant Fish and Game Council.

If at any time DOC considers that the swan nesting numbers have reduced as a result of the concession activity, changes to the spillway height may be required. MDC will implement the required changes as soon as practicable.

## 6.4 Water level monitoring

During the term of the concession MDC are required to carry out water level monitoring to establish the levels of water in the lagoon and at the weir. This monitoring requires the reading and recording of results from two staff gauges in the easement area, the gauges must be read a minimum of five times annually, with the results of these readings being forwarded to DOC annually. To ensure this monitoring requirement is fulfilled MDC utilities staff will carry out bi-monthly site visits to the Lagoon to record the water levels shown on the two gauges.

MDC is also required to note when flooding or ponding occurs on the adjacent Landcorp Farming Ltd property. MDC has arranged for Landcorp Farming Ltd to record periods of inundation by taking photos at set photo points and reading the gauges at the time of inundation. It is proposed that this requirement is expanded to include other adjacent landowners such as those to the south of the lagoon due to the history of inundation during heavy rain events or wet periods of the year.

The existing staff gauge adjacent to the interconnecting drain between the two lakes will be maintained and monitored by MDC.

MDC may wish to utilise a drone in order to capture aerial footage and imagery of the lagoon to allow for the water levels to be monitored and capture any restrictions in the drainage channels which are not easily identified from the ground. In order for this to be done Council must apply for a permit to fly a drone. It is recommended that this be completed in October or prior to and following any vegetation clearance. The drone will be flown over the full extent of the lagoon however the focus will be on the easement areas relating to the concession. As the lagoon is a green aircraft zone a drone can be flown within the site however authorisation is still required from DOC. An application to vary the concession to include the drone flying authorisation will be completed if this is deemed necessary and the Oroua Downs Drainage Scheme has sufficient funding to do so in the future.

## 6.5 Inspections

The table below outlines the proposed annual monitoring/inspection schedule.

| Date/frequency | Task | Requirements |
| --- | --- | --- |
|  | Pre and post survey if mechanical weed removal occurs, joint inspection with DOC following mechanical clearing. | Investigation and documentation to demonstrate that the width and/or depth of the drain has not increased following mechanical weed removal. |
| Bi-Monthly | Council to complete bi-monthly site visits. | - Photograph the margins of the lagoon at set points to help document weed colonisation - Infrastructure survey - Water quality - Water level readings - Confirm fish passage - Landcorp ponding assessment |
| September - Annually | Fish and Game bird count records | Request bird count records from Fish and Game in September each year |
| September | Data assessment in preparation for Hui | Analysis of the bird and weed trend data for discussion with DOC and Rangitāne o Manawatū |
| October - Annually | Hui between MDC, DOC, Ngāti Apa and Rangitāne o Manawatū. | Discussion of the following: - Monitoring results - Any trends - Draft recommendations - Final recommendations |
| November – Annually | Annual report submitted to DOC with copy to Rangitāne o Manawatū. | Annual report to include the following information: - Summary of all monitoring and inspection results If spraying or mechanical weed removal is proposed within the following year the following must be obtained and documented: - Approval from DOC - Written approval from Horizons & DOC if spraying proposed - Confirmation of approved chemicals for the use of spraying. |
| Within 3 weeks of any mechanical clearance | Council and DOC to undertake a post vegetation clearance inspection within three weeks of the clearance. |  |

## 6.6 Annual reporting

MDC will ensure that the data collected as a result of the environmental monitoring is made available to DOC annually, as soon after collection as practicable.

Appendix A – Easement Area Plan